/*******************************************************

* -------------

* NintendOscope

* -------------

* Gameboy --> Oscilloscope adapter using an XMega128A1 + R2R DAC

*

* Connections:

* 		Gameboy HSYNC 	-- PC0

*		Gameboy VSYNC 	-- PA4

*		Gameboy DATA0 	-- PB5 			(MOSI)

*		Gameboy DATA1 	-- PC5			(MOSI)

*		Gameboy CLOCK 	-- PC7 + PD7 	(SCLK)

*		XMega PC1 	-- PC4 + PD4 	(Slave select)

*		XMega PA0	-- R2R DAC input 1

*		XMega PA1	-- R2R DAC input 2

*		SCOPE A input 	-- PA2

*		SCOPE B input 	-- PB2

*		SCOPE Z input 	-- R2R DAC output

*		XMega PE0..3    -- LEDS / Logic analyzer

*

* Craig (flashingleds.wordpress.com)

* 10/02/2011

* Do whatever you want with it

******************************************************/

#include <util/delay.h>

#include <avr/interrupt.h>

#include <avr/io.h>

volatile unsigned char NewFrameHasStarted; 		// Used to flag the start of a new frame draw by the Gameboy (tripped by the VSYNC line)

ISR (PORTA_INT1_vect) //VSYNC

{

NewFrameHasStarted=1;

}

int main (void)

{

unsigned char ii,jj,kk;						// For all your looping needs

// Frame capture variables //

unsigned int TotalBytesCaught; 				// Used to keep track of how many bytes have been captured so we know when we've got a full frame

unsigned char Databit0_Capture_Buffer[2880];// Pixel data arrays.  2880 is (160x144)/8

unsigned char Databit1_Capture_Buffer[2880];//

// Frame raster variables //

unsigned char window_size_x, window_size_y; // Sets the raster DAC ranges

unsigned char temp_db0,temp_db1; 			// Used as a speedup (to avoid accessing Databit0_Capture_Buffer[] every loop iteration)

unsigned int index;							// Keeps track of where you are up to in the pixel data arrays

unsigned char bitposition;					// Keeps track of which bit in the current byte you should display

window_size_x=160;

window_size_y=144;

index=0;

bitposition=0;

/********* CONFIGURE IO PORTS *********/

PORTE.DIR = 0b00001111;		// For status LEDs

PORTE.OUT = 0b00000000;

PORTC.DIR = 0b00000010;		// For the SPI SS control scheme (pins 0 & 1)

PORTC.OUT = 0b00000000;

PORTD.DIR = 0b00000000;

PORTD.OUT = 0b00000000;

PORTA.DIR = 0b00000011;		// For output to the R2R DAC, plus a pullup on the VSYNC interrupt

PORTA.OUT = 0b00010000;

/**************************************/

/**** SETUP INTERNAL 32MHz OSCILLATOR ****/

OSC.CTRL = OSC_RC32MEN_bm; 					// Turn on the oscillator

while(!(OSC.STATUS & OSC_RC32MRDY_bm)) {}	// Wait until it has stabilised

CCP=0xD8;									// Unlock the IO registers via CCP

CLK.CTRL=CLK_SCLKSEL0_bm;					// Switch over the system clock

CCP=0xD8;									// Unlock the IO registers via CCP

CLK.PSCTRL=0b00000000;						// Prescaler A, set clk_per to 32MHz

/******************************************/

/********** SETUP SPI MODULES **************

* 	On the XMega128A1 there is an SPI module

*	available on ports C,D,E,F.

*	Pinout is x7=sck;x5=mosi;x4=ss;x6=miso */

SPIC.CTRL=(SPI_ENABLE_bm|SPI_MODE0_bm);  	//Mode2 (sample on falling edge). SLAVE MODE

SPID.CTRL=(SPI_ENABLE_bm|SPI_MODE0_bm);  	//Mode2 (sample on falling edge). SLAVE MODE

/******************************************/

/********** SETUP INTERRUPTS **************/

PORTA.INT1MASK=0b00010000; 	// Assign PORTA interrupt 1 to pin 4

PORTA.INTCTRL=0b00000100; 	// Set a low-level interrupt

PORTA.PIN4CTRL=0b00000001; 	// trigger on a rising edge

PMIC.CTRL|=(PMIC_LOLVLEX_bm|PMIC_MEDLVLEX_bm|PMIC_HILVLEX_bm); // Enable interrupts of all priority level

sei();

/******************************************/

/********** SETUP DACS **************/

DACA.CTRLB=0; 											// Operate with single channel output

DACA.CTRLC=((1<<DAC_LEFTADJ_bp)|(1<<DAC_REFSEL0_bp)); 	// Left adjust the data input so we can get away with 8bit. Use Avcc as reference.

DACA.TIMCTRL=((1<<DAC_CONINTVAL1_bp)|(1<<DAC_CONINTVAL0_bp)|(1<<DAC_REFRESH3_bp)|(1<<DAC_REFRESH2_bp)|(1<<DAC_REFRESH1_bp)|(1<<DAC_REFRESH0_bp));   //Bare minimum 12clocks (1.5us) between conversions, ASSUMING CLK_PER=8MHz. Autorefresh is OFF

DACA.CTRLA=((1<<DAC_CH0EN_bp)|(1<<DAC_ENABLE_bp)); 		// enable dac and output channel 0

DACB.CTRLB=0; 											// Operate with single channel output

DACB.CTRLC=((1<<DAC_LEFTADJ_bp)|(1<<DAC_REFSEL0_bp)); 	// Left adjust the data input so we can get away with 8bit. Use Avcc as reference.

DACB.TIMCTRL=((1<<DAC_CONINTVAL1_bp)|(1<<DAC_CONINTVAL0_bp)|(1<<DAC_REFRESH3_bp)|(1<<DAC_REFRESH2_bp)|(1<<DAC_REFRESH1_bp)|(1<<DAC_REFRESH0_bp));   //Bare minimum 12clocks (1.5us) between conversions, ASSUMING CLK_PER=8MHz. Autorefresh is OFF

DACB.CTRLA=((1<<DAC_CH0EN_bp)|(1<<DAC_ENABLE_bp)); 		// enable dac and output channel 0

/******************************************/

//----------------------------------------------------------------

// This loop will test the intensity modulation - put the scope in

// regular (timesweep) mode and put the voltage output from the relevant

// DAC to both a scope input and the scope intensity modulation input.

// See the blog post for an example of this.

//----------------------------------------------------------------

/*	while(1)

{

PORTA.OUT=0;

_delay_us(4);

PORTA.OUT=1;

_delay_us(4);

PORTA.OUT=2;

_delay_us(4);

PORTA.OUT=3;

_delay_us(4);

}

*/

//----------------------------------------------------------------

// This loop will test the XY raster by drawing a 160x144 square

//----------------------------------------------------------------

/*

PORTA.OUT=0; // Full intensity

while(1)

{

for(ii=20+window_size_y;ii>20;ii--)

{

DACB.CH0DATAH=ii;

for(jj=20;jj<window_size_x+20;jj++) // The start of the DAC ramp is a little nonlinear, so we start it at 20 instead of 0

{

DACA.CH0DATAH=jj;

_delay_us(1);

}

}

}

*/

//-----------------

//	MAIN LOOP

//-----------------

while(1)

{

// @@@@@@@@@@@ CAPTURE A FRAME @@@@@@@@@@@@@@@@@

PORTA.OUT=0b11; //Blank the spot while we're capturing

// --- FRAME START SYNCHRONIZATION ---

NewFrameHasStarted=0;

TotalBytesCaught=1;		//If I started at 0 I was getting screen artifacts - the first 8 horizontal pixels were being put at the end of the line instead of the start. This seems to fix it, but to be honest I'm not entirely sure why

while(NewFrameHasStarted==0) {}

PORTE.OUTSET = ( 1<< 1); //Signal frame start

while(TotalBytesCaught<2880) // 2880 = number of bytes in 160x144 pixels

{

// --- LINE START SYNCHRONIZATION ---

// You could think about using the HSYNC pulse itself as the SS control for the SPI modules,

// the only problem is that it takes too long. Instead I've set up a rising edge triggered

// pulse. Wait for the HSYNC pulse on PORTC.0, then after that rising edge immediately pull

// PORTC.1 low to enable the SPI modules

PORTC.OUTSET=(1<<1); // Hold the SPI modules in reset

while(!(PORTC.IN & (1<<0))) // Do nothing until you get an HSYNC pulse

asm volatile("nop");

PORTC.OUTCLR=(1<<1);  // Enable the SPI modules (HSYNC is still high at this point)

for(kk=0;kk<20;kk++) // Catch 20 bytes (one line = 160 pixels = 20 bytes)

{

while(!(SPID.STATUS & SPI_IF_bm)){}

Databit0_Capture_Buffer[TotalBytesCaught]=SPID.DATA;

Databit1_Capture_Buffer[TotalBytesCaught]=SPIC.DATA;

TotalBytesCaught++;

}

PORTE.OUTTGL = ( 1<< 0); //Signal line complete

}

PORTE.OUTCLR = ( 1<< 1); //Signal frame complete

// @@@@@@@@@@@ DRAW THE FRAME @@@@@@@@@@@@@@@@@

index=0;

bitposition=0;

PORTE.OUTSET = (1<<2);  //Signal raster start

for(ii=20+window_size_y;ii>20;ii--)

{

DACB.CH0DATAH=ii; // Increment Y-deflection DAC

temp_db0=Databit0_Capture_Buffer[index];

temp_db1=Databit1_Capture_Buffer[index];

for(jj=window_size_x+20;jj>20;jj--) // The start of the DAC ramp is a little nonlinear, so we start it at 20 instead of 0

{

DACA.CH0DATAH=jj; // Increment X-deflection DAC

// Set the output for the R2R DAC on PORTA

if((temp_db1 & bitposition))

PORTA.OUTSET=0b00000001;

else

PORTA.OUTCLR=0b00000001;

if((temp_db0 & bitposition))

PORTA.OUTSET=0b00000010;

else

PORTA.OUTCLR=0b00000010;

// Delay loop, for sub-1us delay resolution. This one gives about 1us

for(kk=0;kk<4;kk++)

{

asm volatile("nop");

asm volatile("nop");

}

bitposition=bitposition>>1;

if(bitposition==0) // You've displayed 8 pixels and need to reload temp_db0 & temp_db1

{

bitposition=128;

index++;

temp_db0=Databit0_Capture_Buffer[index];

temp_db1=Databit1_Capture_Buffer[index];

}

}

PORTE.OUTTGL = ( 1<< 3); //Signal end of raster line

}

PORTE.OUTCLR = (1<<2);  //Signal raster end

}

return 0;

}